Workcell自動化專案之PLC功能開發與通訊協定串接

- 簡介

本委託案為完成2組Workcell自動化加工產線之PLC自動化控制程式與其相關自動化功能。共有2組Workcell之工作需完成。第一組Workcell，包括1台車床與1台手臂，需完成車床機台控制器內部PLC邏輯開發，與機械手臂上下料之通訊協定串接功能。第二組Workcell，包括1台立式銑床與1台歐姆龍手臂，需完成立式銑床機台控制器與歐姆龍機械手臂上下料之通訊協定串接功能。

- 規格

- 第一組Workcell：

第一組Workcell，包括1台車床與1台手臂，需完成車床機台控制器內部PLC程式開發，與機械手臂上下料之通訊協定串接功能。

- 開發車床機台之PLC控制程式

  - 開發方式依照操作面板之48/32 I/O卡與96/64 I/O卡之I/O定義，再依照控制器之C點、S點與Reg點定義(請見七.)來編寫車床機台之PLC控制程式。

  - PLC程式功能包括操作面板上所有按鈕與控制器之相關所有功能，確保以下功能需正常無誤，功能如下

- MDI、AUTO、EDIT、HOME、MPG等模式切換

- 吋動\連續移動切換

- 各軸向JOG功能

- C/S軸切換

- CYCLE START/HOLD/RESET

- 主軸正轉、停止、反轉

- 主軸轉動轉速百分比

- 連續移動速度百分比

- 加工進給速度百分比

- SBK/BDT/OSP/MLK功能

- 刀塔正轉/反轉、刀塔回原點

- 極限開關、緊急停止、伺服啟動等功能

- 原點DOG功能

- 機台狀態與三色燈信號

- MPG功能：1X、10X、100X

- 開發機械手臂之上下料之通訊協定串接之PLC功能

PLC程式功能包括如下

- M88交換料功能

- M89純上料功能

- 上料位置設定

- 下料位置設定

- 機械手臂調機

- 第二組Workcell：

- 依照下圖之動作程式碼開發機械手臂之上下料之通訊協定串接之銑床控制器中的PLC功能

PLC程式功能包括如下

- M88交換料功能

- M89純上料功能

- 上料位置設定

- 下料位置設定

- 機械手臂調機

- 工作內容

- 第一組Workcell：

工研院：

- 提供機台與控制器之C點、S點與Reg點定義(請見七.)

- 提供PLC之Ladder編輯軟體。

得標廠商：

- 依照『規格之第一組Workcell』所述，完成開發車床機台之PLC控制程式。

- 依照『規格之第一組Workcell』所述，完成開發機械手臂之上下料之通訊協定串接之PLC功能。

- 實際工廠產線進車床機台與機械手臂上下料之PLC控制程式測試與驗證，需驗證所有功能皆為正常無誤。。

- 提供繳交所開發完成並完成驗證之PLC程式原始碼。

- 將開發完之PLC程式Ladder檔，複製到車床機台控制器中，進行功能測試，確保車床功能與機械手臂上下料皆為正常。測試功能項目如規格所列，並以下表之驗證項目作確認與紀錄，確保所有功能皆為正常無誤。

| 編號 | 功能項目 | 功能是否正常 |
| --- | --- | --- |
| 1 | MDI、AUTO、EDIT、HOME、MPG等模式切換 |  |
| 2 | 吋動\連續移動切換 |  |
| 3 | 各軸向JOG功能 |  |
| 4 | C/S軸切換 |  |
| 5 | CYCLE START/HOLD/RESET |  |
| 6 | 主軸正轉、停止、反轉 |  |
| 7 | 主軸轉動轉速百分比 |  |
| 8 | 連續移動速度百分比 |  |
| 9 | 加工進給速度百分比 |  |
| 10 | SBK/BDT/OSP/MLK功能 |  |
| 11 | 刀塔正轉/反轉、刀塔回原點 |  |
| 12 | 極限開關、緊急停止、伺服啟動等功能 |  |
| 13 | 原點DOG功能 |  |
| 14 | 機台狀態與三色燈信號 |  |
| 15 | MPG功能：1X、10X、100X |  |
| 16 | M88交換料功能 |  |
| 17 | M89純上料功能 |  |
| 18 | 上料位置設定 |  |
| 19 | 下料位置設定 |  |
| 20 | 機械手臂調機 |  |

- 第二組Workcell：

工研院：

- 提供銑床機台之IO點與機械手臂之IO點定義(請見八.)

- 提供銑床PLC之Ladder編輯軟體。

得標廠商：

- 依照『規格之第二組Workcell』所述，完成開發歐姆龍機械手臂之上下料之通訊協定串接之PLC功能。

- 實際工廠產線進銑床機台與歐姆龍機械手臂上下料之PLC控制程式測試與驗證，需驗證所有功能皆為正常無誤。

- 提供繳交所開發完成並完成驗證之PLC程式原始碼。

- 將開發完之PLC程式Ladder檔，複製到銑床機台控制器中，進行功能測試，確保銑床功能與機械手臂上下料皆為正常。測試功能項目如規格所列，並以下表之驗證項目作確認與紀錄，確保所有功能皆為正常無誤。

| 編號 | 功能項目 | 功能是否正常 |
| --- | --- | --- |
| 1 | M88交換料功能 |  |
| 2 | M89純上料功能 |  |
| 3 | 上料位置設定 |  |
| 4 | 下料位置設定 |  |
| 5 | 機械手臂調機 |  |

- 委託交付

- 繳交開發完成並完成驗證之第一組Workcell之PLC Ladder程式原始碼。

- 繳交第一組Workcell之PLC Ladder程式功能驗證之表格。

- 繳交開發完成並完成驗證之第二組Workcell之PLC Ladder程式原始碼。

- 繳交第二組Workcell之PLC Ladder程式功能驗證之表格。

- 驗收方式與標準

- 驗收方式：依照『四.委託交付』所列，分別繳交PLC Ladder電子檔與驗收檢驗表，再依照『三. 工作內容』所列之所有項目，進行規格功能檢驗，確認是否符合所列之功能項目，驗收項目請見驗收標準所述，若有任何一項規格不符合，即為不合格。

(二) 驗收標準：是否符合『二. 規格』中所列的每一項功能規格，並以『三. 工作內容』中所述之驗收功能表格進行每一項功能之驗收確認，必須每一項都合乎要求才能達驗收成功之標準。

- 保固

保固服務是以檔案交付後一年為期限，期間內如軟體功能發生非人為之異常或錯誤，必須無條件免費除錯，若在保固期外，則另計價維修。

- 車床 C,S Bits和暫存器

    - C BITS定義

C BIT 	 MLCCNC

| BIT # | SYMBOL | DESCRIPTION | NOTE |
| --- | --- | --- | --- |
| 000 | ST | CYCLE START |  |
| 001 | SP | FEED HOLD |  |
| 002 | SKIP | G31 SKIP SIGNAL |  |
| 003 | EDTD2S | PROGRAM PROTECTION |  |
| 004 | manual_ABS | Manual Absolute Signal |  |
| 005 | ErrorDetect | Reserverd |  |
| 006 | +X | +X AXIS DIRECTION |  |
| 007 | -X | -X AXIS DIRECTION |  |
| 008 | +Y | +Y AXIS DIRECTION |  |
| 009 | -Y | -Y AXIS DIRECTION |  |
| 010 | +Z | +Z AXIS DIRECTION |  |
| 011 | -Z | -Z AXIS DIRECTION |  |
| 012 | +4 | +4 AXIS DIRECTION |  |
| 013 | -4 | -4 AXIS DIRECTION |  |
| 014 |  |  |  |
| 015 | HG | HANDLE GUIDANCE |  |
| 016 | HX | HANDLE X AXIS |  |
| 017 | HY | HANDLE Y AXIS |  |
| 018 | HZ | HANDLE Z AXIS |  |
| 019 | H4 | HANDLE 4 AXIS |  |
| 020 | MPGDRN | MPG DRY RUN |  |
| 021 | 保留 |  |  |
| 022 | 保留 |  |  |
| 023 | RT | RAPID TRAVERSE |  |
| 024 | SRVOFF | SERVO OFF |  |
| 025 | Reserverd | Reserverd |  |
| 026 | Reserverd | Reserverd |  |
| 027 | Reserverd | Reserverd |  |
| 028 | Reserverd | Reserverd |  |
| 029 |  |  |  |
| 030 |  |  |  |
| 031 | +5 | +5 AXIS DIRECTION |  |
| 032 | -5 | -5 AXIS DIRECTION |  |
| 033 | +6 | +6 AXIS DIRECTION |  |
| 034 | -6 | -6 AXIS DIRECTION |  |
| 035 |  |  |  |
| 036 | ESP | EMERGENCY STOP |  |
| 037 | ERS | EXTERNAL RESET |  |
| 038 | FIN | M,S,T FINISH |  |
| 039 | SPCMD | 主軸旋轉中切換模式聽自動命令 |  |
| 040 | SBK | SINGLE BLOCK |  |
| 041 | BDT | OPTIONAL BLOCK SKIP |  |
| 042 | DRN | DRY RUN |  |
| 043 | MLK | MACHINE LOCK |  |
| 044 | OPS | OPTIONAL STOP |  |
| 045 | ZNG | Z AXIS CANCEL |  |
| 046 | AFL | AUXILIARY FUNCTION LOCK |  |
| 047 | AF1 | Reserverd |  |
| 048 | AF2 | Reserverd |  |
| 049 | 4thNG | Reserverd |  |
| 050 | +LX | +X AXIS OT |  |
| 051 | -LX | -X AXIS OT |  |
| 052 | +LY | +YAXIS OT |  |
| 053 | -LY | -YAXIS OT |  |
| 054 | +LZ | +ZAXIS OT |  |
| 055 | -LZ | -ZAXIS OT |  |
| 056 | +L4 | +4th AXIS OT |  |
| 057 | -L4 | -4th AXIS OT |  |
| 058 |  | CNC_Storke_Limit2 |  |
| 059 |  | CNC_InterLock_Sgl.x |  |
| 060 |  | CNC_InterLock_Sgl.y |  |
| 061 |  | CNC_InterLock_Sgl.z |  |
| 062 |  | CNC_InterLock_Sgl.ch4 |  |
| 063 |  | CNC_follow_up |  |
| 064 | NA | RESERVED , FOR UERRO手動刀長量測功能 |  |
| 065 | *ITL | All Axes InterLock |  |
| 066 | *CSL | Cutting Block Satrt InterLock |  |
| 067 | *BSL | Block Satrt InterLock |  |
| 068 | TLM | TORQUE LIMIT MANUAL |  |
| 069 | SKIP2 | TORQUE LIMIT SKIP SIGNAL |  |
| 070 | DOGX | X AXIS DOG | EtherCAT |
| 071 | DOGY | Y AXIS DOG | EtherCAT |
| 072 | DOGZ | Z AXIS DOG | EtherCAT |
| 073 |  |  |  |
| 074 |  |  |  |
| 075 | SVAX | X AXIS SERVO ALARM |  |
| 076 | SVAY | Y AXIS SERVO ALARM |  |
| 077 | SVAZ | Z AXIS SERVO ALARM |  |
| 078 | SVA4 | 4th AXIS SERVO ALARM |  |
| 079 | SVA5 | 5th AXIS SERVO ALARM |  |
| 080 | MAIN_HIGHGR | MAIN SPINDLE HIGH GEAR |  |
| 081 | Reserverd | Reserverd |  |
| 082 | Reserverd | Reserverd |  |
| 083 | Reserverd | Reserverd |  |
| 084 | Reserverd | Reserverd |  |
| 085 | MAIN_ORT | MAIN SPINDLE ORIENTATION |  |
| 086 | Reserverd | Reserverd |  |
| 087 | Reserverd | Reserverd |  |
| 088 | Reserverd | Reserverd |  |
| 089 |  | CNC_Mirror_Image.x |  |
| 090 |  | CNC_Mirror_Image.y |  |
| 091 |  | CNC_Mirror_Image.z |  |
| 092 |  | CNC_Mirror_Image.ch4 |  |
| 093 |  | CNC_Mirror_Image.ch5 |  |
| 094 |  | CNC_Mirror_Image.ch6 |  |
| 095 | AUTOGR | AUTO GEAR FUNCTION |  |
| 096 | MANUDA | Spindle DA output for manual mode |  |
| 097 | MAIN_3RDGR | MAIN SPINDLE 3RD GEAR |  |
| 098 | EN_TMONTLM | Enable Torque Monitoring on Tool Life Management |  |
| 099 |  |  |  |
| 100 | UI0 | MACRO INPUT 0 |  |
| 101 | UI1 | MACRO INPUT 1 |  |
| 102 | UI2 | MACRO INPUT 2 |  |
| 103 | UI3 | MACRO INPUT 3 |  |
| 104 | UI4 | MACRO INPUT 4 |  |
| 105 | UI5 | MACRO INPUT 5 |  |
| 106 | UI6 | MACRO INPUT 6 |  |
| 107 | UI7 | MACRO INPUT 7 |  |
| 108 | UI8 | MACRO INPUT 8 |  |
| 109 | UI9 | MACRO INPUT 9 |  |
| 110 | UI10 | MACRO INPUT 10 |  |
| 111 | UI11 | MACRO INPUT 11 |  |
| 112 | UI12 | MACRO INPUT 12 |  |
| 113 | UI13 | MACRO INPUT 13 |  |
| 114 | UI14 | MACRO INPUT 14 |  |
| 115 | UI15 | MACRO INPUT 15 |  |
| 116 | MTC | MANUAL TOOL CHANGE |  |
| 117 | 保留 | MILLING ON |  |
| 118 | MAIN_MIDGR | MAIN SPINDLE MIDDLE GEAR |  |
| 119 | ZP2ZDC | ZP2Z DOWN CANCEL |  |
| 120 | 保留 | PMC AXIS GO |  |
| 121 | MAIN_LOWGR | MAIN SPINDLE LOW GEAR |  |
| 122 | APPR | APPROACH |  |
| 123 | GUIDE | GUIDANCE |  |
| 124 | SRAMW | SRAM WRITE |  |
| 125 | MAIN_RIGTAP | MAIN SPINDLE RIGID TAPPING |  |
| 126 | MAIN_ORTS | MAIN SPINDLE ORIENTATION SENSOR |  |
| 150 | MAIN_CF2CF | MAIN SPINDLE CS/CF AXIS CHANGE TO C AXIS FINISH |  |
| 151 | 保留(立車版本) | CF軸模式，第四軸SERVO ON信號 |  |
| 152 | +L5 | +5th AXIS OT |  |
| 153 | -L5 | -5th AXIS OT |  |
| 154 | +L6 | +6th AXIS OT |  |
| 155 | -L6 | -6th AXIS OT |  |
| 156 | 保留 | CNC_InterLock_Sgl.ch5 |  |
| 157 | 保留 | CNC_InterLock_Sgl.ch6 |  |
| 158 | SVA6 | 6th AXIS SERVO ALARM |  |
| 159 |  |  |  |
| 160 | MAIN_SPD1 | MAIN SPINDLE 1 IS SELECTED |  |
| 161 | MILL_SPD2 | MILL SPINDLE 2 IS SELECTED |  |
| 162 | EPOC | External Power Off Command |  |
| 163 |  | TailStock Pin On |  |
| 164 |  | TailStock Center On |  |
| 165 | SRFTC | Spindle Run for Tool Change |  |
| 166 |  | X Axis GO HOME COMMAND |  |
| 167 |  | Y Axis GO HOME COMMAND |  |
| 168 |  | X Axis GO HOME FIRST |  |
| 169 | MAIN_CUTCHK | Cancel Check MAIN spindle speed arrival when it is G00 command |  |
| 170 | MCGSPD1 | Manual Change Gear of Spindle 1 |  |
| 171 | M19_Servo.ON | 第一主軸伺服啟動 Servo on |  |
| 172 | M19Command | 第一主軸開始定位流程 Start flow |  |
| 173 | 手動正轉 | 當輸入為+-10v時 主軸正轉訊號 |  |
| 174 | 手動反轉 | 當輸入為+-10v時 主軸反轉訊號 |  |
| 175 | M29_Servo.ON | 第二主軸伺服啟動Servo on |  |
| 176 | M29Command | 第二主軸開始定位流程 Start flow |  |
| 177 | MAIN_DigiSVon | 全數位主軸Servo On |  |
| 178 | MILL_DigiSVon | 全數位銑削主軸Servo On |  |
| 179 | Dy_BK_ON | 軸向煞車信號(煞車->ON) |  |
| 180 | Mill_Reverse | 銑削軸命令反向 |  |
| 181 | Mill_LOWGR | Mill SPINDLE LOW GEAR |  |
| 182 | Mill _MIDGR | Mill SPINDLE MIDDLE GEAR |  |
| 183 | Mill _3RDGR | Mill SPINDLE 3RD GEAR |  |
| 184 | Mill _HIGHGR | Mill SPINDLE HIGH GEAR |  |
| 187 | MCGMillSPD | Manual Change Gear of Mill Spindle |  |
| 190 | SP_ORI_TC | SP Orientation Tool Change |  |
| 191 | SP1_JOG | 主軸吋動 |  |
| 193 | SP2_JOG | 銑銷軸吋動 |  |
| 195 | SP3_JOG | 副主軸吋動 |  |
| 200 | SK_ST | Cycle Start Light On | SoftPanel |
| 201 | SK_HOLD | Feed Hold Light On | SoftPanel |
| 202 | SK_RESET | Reset Light On | SoftPanel |
| 203 |  | (HMICNC) | SoftPanel |
| 204 |  | (HMICNC) | SoftPanel |
| 205 | SK_POWER | Power Light On(SoftPanel) | SoftPanel |
| 206 | SK_LIGHT | Lighting Light On(SoftPanel) | SoftPanel |
| 207 | SK_AIR | Air Coolant Light On(SoftPanel) | SoftPanel |
| 208 | SK_SHOWER | Shower Light On(SoftPanel) | SoftPanel |
| 209 | SK_FLUSH | Flushing Light On(SoftPanel) | SoftPanel |
| 210 | SK_CTS | 刀具中心出水Light On(SoftPanel) | SoftPanel |
| 211 | SK_DOOR | Door Open Light On(SoftPanel) | SoftPanel |
| 212 | SK_SBK | Single Block Light On(SoftPanel) | SoftPanel |
| 213 | SK_OSP | Option Stop Light On(SoftPanel) | SoftPanel |
| 214 | SK_BDT | Option Block Skip Light On(SoftPanel) | SoftPanel |
| 215 | SK_MLK | Machine Lock Light On(SoftPanel) | SoftPanel |
| 216 | SK_Z MLK | Z.Cancel Light On(SoftPanel) | SoftPanel |
| 217 | SK_MST // | M.S.T Lock Light On(SoftPanel) | SoftPanel |
| 218 | SK_DRN | Dry Run Light On(SoftPanel) | SoftPanel |
| 219 | SK_MPG DRN | MPG Dry Run Light On(SoftPanel) | SoftPanel |
| 220 | SK_ATC | Tool Change Light On(SoftPanel) | SoftPanel |
| 221 | SK_ORT | Spindle Orientation Light On(SoftPanel) | SoftPanel |
| 222 |  | (HMICNC) | SoftPanel |
| 223 |  | (HMICNC) | SoftPanel |
| 224 |  | (HMICNC) | SoftPanel |
| 225 |  | (HMICNC) | SoftPanel |
| 226 |  | (HMICNC) | SoftPanel |
| 227 |  | (HMICNC) | SoftPanel |
| 228 |  | (HMICNC) | SoftPanel |
| 229 |  | (HMICNC) | SoftPanel |
| 230 |  | (HMICNC) | SoftPanel |
| 231 |  | (HMICNC) | SoftPanel |
| 232 | SK_FEED 100% | Feed Overrate 100% Light On(SoftPanel) | SoftPanel |
| 233 |  | (HMICNC) | SoftPanel |
| 234 |  | (HMICNC) | SoftPanel |
| 235 | SK_RAPID 100% | Rapid 100% Light On(SoftPanel) | SoftPanel |
| 236 |  | (HMICNC) | SoftPanel |
| 237 |  | (HMICNC) | SoftPanel |
| 238 | SK_SP 100% | Spindle overrate 100% Light On(SoftPanel) | SoftPanel |
| 239 | SK_MPG*1 | MPG Override X1 Light On(SoftPanel) | SoftPanel |
| 240 | SK_MPG*10 | MPG Override X10 Light On(SoftPanel) | SoftPanel |
| 241 | SK_MPG*100 | MPG Override X100 Light On(SoftPanel) | SoftPanel |
| 242 | -X | -X Axis Direction | SoftPanel |
| 243 | +X | +X Axis Direction | SoftPanel |
| 244 | -Y | -Y Axis Direction | SoftPanel |
| 245 | +Y | +Y Axis Direction | SoftPanel |
| 246 | -Z | -Z Axis Direction | SoftPanel |
| 247 | +Z | +Z Axis Direction | SoftPanel |
| 248 | -4 | -4 Axis Direction | SoftPanel |
| 249 | +4 | +4 Axis Direction | SoftPanel |
| 250 | -5 | -5 Axis Direction | SoftPanel |
| 251 | +5 | +5 Axis Direction | SoftPanel |
| 252 |  | (HMICNC) | SoftPanel |
| 253 |  | (HMICNC) | SoftPanel |
| 254 |  | (HMICNC) | SoftPanel |
| 255 |  | (HMICNC) | SoftPanel |
| 256 |  | (HMICNC) | SoftPanel |
| 257 |  | (HMICNC) | SoftPanel |
| 258 |  | (HMICNC) | SoftPanel |
| 259 |  | (HMICNC) | SoftPanel |
| 260 |  | (HMICNC) | SoftPanel |
| 261 |  | (HMICNC) | SoftPanel |
| 262 |  | (HMICNC) | SoftPanel |
| 263 |  | (HMICNC) | SoftPanel |
| 264 |  | (HMICNC) | SoftPanel |
| 265 |  | (HMICNC) | SoftPanel |
| 266 |  | (HMICNC) | SoftPanel |
| 267 |  | (HMICNC) | SoftPanel |
| 268 |  | (HMICNC) | SoftPanel |
| 269 |  | (HMICNC) | SoftPanel |
| 270 |  | (HMICNC) | SoftPanel |
| 271 |  | (HMICNC) | SoftPanel |
| 272 |  | (HMICNC) | SoftPanel |
| 273 |  | (HMICNC) | SoftPanel |
| 274 |  | (HMICNC) | SoftPanel |
| 275 |  | (HMICNC) | SoftPanel |
| 276 |  | (HMICNC) | SoftPanel |
| 277 |  | (HMICNC) | SoftPanel |
| 278 |  | (HMICNC) | SoftPanel |
| 279 |  | (HMICNC) | SoftPanel |
| 280 | SK_SP_CW | Spindle Forward Light On(SoftPanel) | SoftPanel |
| 281 | SK_SP_STP | Spindle Stop Light On(SoftPanel) | SoftPanel |
| 282 | SK_SP_CCW | Spindle Reverse Light On(SoftPanel) | SoftPanel |
| 283 | SK_ATC CW | ATC motor cw Light On(SoftPanel) | SoftPanel |
| 284 | SK_ATC CCW | ATC motor ccw Light On(SoftPanel) | SoftPanel |
| 285 | SK_CHIP ON | 捲屑機正轉Light On(SoftPanel) | SoftPanel |
| 286 | SK_CHIP REV | 捲屑機反轉Light On(SoftPanel) | SoftPanel |
| 287 | SK_Water ON | Coolant Fluid On Light On(SoftPanel) | SoftPanel |
| 288 | SK_Water STP | Coolant Fluid Stop Light On(SoftPanel) | SoftPanel |
| 289 | SK_Water AUTO | Coolant Fluid Auto Light On(SoftPanel) | SoftPanel |
| 290 | SK_EDIT | EDIT Mode Light On(SoftPanel) | SoftPanel |
| 291 | SK_MEM | MEM Mode Light On(SoftPanel) | SoftPanel |
| 292 | SK_MDI | MDI Mode Light On(SoftPanel) | SoftPanel |
| 293 | SK_JOG | Jog Mode Light On(SoftPanel) | SoftPanel |
| 294 | SK_RAPID | Rapid Mode Light On(SoftPanel) | SoftPanel |
| 295 | SK_MPG | MPG Mode Light On(SoftPanel) | SoftPanel |
| 296 | SK_HOME | Home Mode Light On(SoftPanel) | SoftPanel |
| 297 | SK_OPR | OPER Mode Light On(SoftPanel) | SoftPanel |
| 298 |  | (HMICNC) | SoftPanel |
| 299 |  | (HMICNC) | SoftPanel |
| 300 |  | 主軸吋動 |  |
| 310 |  | 銑銷軸吋動 |  |
| 320 |  | 副主軸吋動 |  |
| 350 | 保留(噴漆機) | 輸送帶轉動 |  |
| 351 | 保留(噴漆機) | 輸送帶上物體追蹤模式 |  |
| 352 | 保留(噴漆機) | 往復機模式 |  |
| 353 | 保留(噴漆機) |  |  |
| 354 | 保留(噴漆機) |  |  |
| 355 | 保留(噴漆機) | 開啟DAC1輸出(扇形噴槍) |  |
| 356 | 保留(噴漆機) | 開啟DAC2輸出(霧化噴槍) |  |
| 357 | 保留(噴漆機) | 開啟DAC3輸出(第一支槍流量) |  |
| 358 | 保留(噴漆機) | 開啟DAC4輸出(第二支槍流量) |  |
| 359 | 保留(噴漆機) | 開啟DAC5輸出(第三支槍流量) |  |
| 360 | 保留(噴漆機) | 開啟DAC6輸出(第四支槍流量) |  |
| 361 | 保留(噴漆機) | Optical Sensor 光感測器感測物體 |  |
| 362 | 保留(噴漆機) | 設定往復機的機械座標 |  |
| 400 | 保留 | 保留 |  |
| 401 | 保留 | CNC_InterLock_P_Sgl.x |  |
| 402 | 保留 | CNC_InterLock_P_Sgl.y |  |
| 403 | 保留 | CNC_InterLock_P_Sgl.z |  |
| 404 | 保留 | CNC_InterLock_P_Sgl.ch4 |  |
| 405 | 保留 | CNC_InterLock_P_Sgl.ch5 |  |
| 406 | 保留 | CNC_InterLock_P_Sgl.ch6 |  |
| 420 | 保留 | CNC_InterLock_N_Sgl.x |  |
| 421 | 保留 | CNC_InterLock_N_Sgl.y |  |
| 422 | 保留 | CNC_InterLock_N_Sgl.z |  |
| 423 | 保留 | CNC_InterLock_N_Sgl.ch4 |  |
| 424 | 保留 | CNC_InterLock_N_Sgl.ch5 |  |
| 425 | 保留 | CNC_InterLock_N_Sgl.ch6 |  |
| 430 | SP1_Ori_Short | 第一主軸採最短距離定位 |  |
| 431 | SP2_Ori_Short | 第二主軸採最短距離定位 |  |
| 432 | SP3_Ori_Short | 第三主軸採最短距離定位 |  |
| 433 |  |  |  |
| 434 | 1SP_CW | 第一主軸正轉訊號(保留) |  |
| 435 | 2SP_CW | 第二主軸正轉訊號(保留) |  |
| 436 | 3SP_CW | 第三主軸正轉訊號(保留) |  |
| 437 | 4SP_CW | 第四主軸正轉訊號(保留) |  |
| 438 | 1SP_CCW | 第一主軸反轉訊號(保留) |  |
| 439 | 2SP_CCW | 第二主軸反轉訊號(保留) |  |
| 440 | 3SP_CCW | 第三主軸反轉訊號(保留) |  |
| 441 | 4SP_CCW | 第四主軸反轉訊號(保留) |  |
| 442 | 1SP_STP | 第一主軸靜止訊號(保留) |  |
| 443 | 2SP_STP | 第二主軸靜止訊號(保留) |  |
| 444 | 3SP_STP | 第三主軸靜止訊號(保留) |  |
| 445 | 4SP_STP | 第四主軸靜止訊號(保留) |  |
| 446 |  |  |  |
| 447 |  |  |  |
| 448 | 保留 |  |  |
| 449 | 保留 |  |  |
| 450 | 1SP_Ori_S | 第一主軸定位偏移值根據主軸端或馬達端(ON-馬達端) |  |
| 451 | 2SP_Ori_S | 第二主軸定位偏移值根據主軸端或馬達端(ON-馬達端) |  |
| 452 | 3SP_Ori_S | 第三主軸定位偏移值根據主軸端或馬達端(ON-馬達端) |  |
|  |  | (保留) |  |
| 454 | 1SP_Ori_ABRE | 相對/絕對第一主軸定位功能選擇 |  |
| 455 | 2SP_Ori_ABRE | 相對/絕對第二主軸定位功能選擇 |  |
| 456 | 3SP_Ori_ABRE | 相對/絕對第三主軸定位功能選擇 |  |
|  |  | (保留) |  |
| 479 | 保留 | 保留給執行M00/M01主軸停止旋轉 |  |
| 480 | SKIP01 | G31 SKIP SIGNAL 01 | 參考C2 |
| 481 | SKIP02 | G31 SKIP SIGNAL 02 |  |
| 482 | SKIP03 | G31 SKIP SIGNAL 03 |  |
| 483 | SKIP04 | G31 SKIP SIGNAL 04 |  |
| 484 | 保留 | G31 SKIP SIGNAL 05 |  |
| 485 | 保留 | G31 SKIP SIGNAL 06 |  |
| 486 | 保留 | G31 SKIP SIGNAL 07 |  |
| 487 | 保留 | G31 SKIP SIGNAL 08 |  |
| 488 | 保留 | G31 SKIP SIGNAL 09 |  |
| 489 | 保留 | G31 SKIP SIGNAL 10 |  |
| 490 | DS01 | DEWELL SKIP SIGNAL 01 |  |
| 491 | DS02 | DEWELL SKIP SIGNAL 02 |  |
| 492 | DS03 | DEWELL SKIP SIGNAL 03 |  |
| 493 | DS04 | DEWELL SKIP SIGNAL 04 |  |
| 494 | 保留 | DEWELL SKIP SIGNAL 05 |  |
| 495 | 保留 | DEWELL SKIP SIGNAL 06 |  |
| 496 | 保留 | DEWELL SKIP SIGNAL 07 |  |
| 497 | 保留 | DEWELL SKIP SIGNAL 08 |  |
| 498 | 保留 | DEWELL SKIP SIGNAL 09 |  |
| 499 | 保留 | DEWELL SKIP SIGNAL 10 |  |
| 500 | MAIN_CHUK_ OK | MAIN Spindle chuck status check ok |  |
| 501 | SUB_CHUK_ OK | SUB Spindle chuck status check ok |  |
| 502 | H5 | HANDLE 5 AXIS |  |
| 503 | H6 | HANDLE 6 AXIS |  |
| 504 | DOG5 | 5 AXIS DOG | EtherCAT |
| 505 | DOG6 | 6 AXIS DOG | EtherCAT |
| 506 | SUB_LOWGR | SUB SPINDLE LOW GEAR |  |
| 507 | SUB_MIDGR | SUB SPINDLE MIDDLE GEAR |  |
| 508 | SUB_3RDGR | SUB SPINDLE 3RD GEAR |  |
| 509 | SUB_HIGHGR | SUB SPINDLE HIGH GEAR |  |
| 510 | SUB_ORT | SUB SPINDLE ORIENTATION |  |
| 511 | SUB_RIGTAP | SUB SPINDLE RIGID TAPPING |  |
| 512 | SUB_ORTS | SUB SPINDLE ORIENTATION SENSOR |  |
| 513 | SUB_CF2CF | SUB SPINDLE CF AXIS CHANGE TO C2 AXIS FINISH |  |
| 514 | SUB_2CFFIN 保留(立車版本) | CF軸模式，第五軸SERVO ON信號 |  |
| 515 | SUB_SPD | SUB SPINDLE IS SELECTED(SUB SPINDLE) |  |
| 516 | SUB_CUTCHK | Check SUB spindle speed arrival when it is cutting command |  |
| 517 | MCGSUBSPD | Manual Change Gear of SUB Spindle |  |
| 518 | SUB_M119_Servo.ON | SUB SPINDLE M119_Servo.ON |  |
| 519 | SUB_M119Command | SUB SPINDLE M119 Command |  |
| 520 | SUB_DigiSVon | Digital Sub Spindle Servo On |  |
| 521 | MAIN_SUB_SEL | Main or Sub Spindle is selected (0:Main/1:Sub) |  |
| 522 | SYN_RDY | Dual spindle synchronized ready |  |
| 523 | ABS_X_ZRN_SET | Absolute X zero point set | 2014.8.18 Add |
| 524 | ABS_Y_ZRN_SET | Absolute Y zero point set | 2014.8.18 Add |
| 525 | ABS_Z_ZRN_SET | Absolute Z zero point set | 2014.8.18 Add |
| 526 | ABS_ZS_ZRN_SET | Absolute Zs zero point set | 2014.8.18 Add |
| 530 | MAIN SP I Gain Freeze | Main Spindle I gain Freeze signal | 2016.10.14Add |
| 531 | SUB SP I Gain Freeze | Sub Spindle I gain Freeze signal | 2016.10.14Add |
| 532 | 2nd Gain switch | Cross Couple controller 2nd gain switch | 2016.10.14Add |
| 533 | Cancel Phs Err Alarm | Cancel Phase Error Alarm Function | 2016.10.14Add |
| 540 | EN_SMP_SP_SYNC | 致能簡易主軸同期 |  |
| 550 | CNC_SV_SYNC1 | Axis Synchronous Control Signal |  |
| 551 | CNC_SV_SYNC2 | Axis Synchronous Control Signal |  |
| 552 | CNC_SV_SYNC3 | Axis Synchronous Control Signal |  |
| 553 | CNC_SV_SYNC4 | Axis Synchronous Control Signal |  |
| 554 | CNC_SV_SYNC5 | Axis Synchronous Control Signal |  |
| 555 | CNC_SV_SYNC6 | Axis Synchronous Control Signal |  |
| 556 | CNC_SV_SYNCJ1 | Manual Feed Axis for Axis Synchronous Control Signal |  |
| 557 | CNC_SV_SYNCJ2 | Manual Feed Axis for Axis Synchronous Control Signal |  |
| 558 | CNC_SV_SYNCJ3 | Manual Feed Axis for Axis Synchronous Control Signal |  |
| 559 | CNC_SV_SYNCJ4 | Manual Feed Axis for Axis Synchronous Control Signal |  |
| 560 | CNC_SV_SYNCJ5 | Manual Feed Axis for Axis Synchronous Control Signal |  |
| 561 | CNC_SV_SYNCJ6 | Manual Feed Axis for Axis Synchronous Control Signal |  |
| 562 | CNC_SV_NSYNCA | Disable torque difference alarm detection for Axis Synchronous Control |  |
| 570 | LNING | 忽略學習訊號 |  |
| 571 | LNMOD | 學習模式 |  |
| 572 | LNOPM | 操作模式選擇 |  |
| 618 | SP1_BRK_Sig | 第一主軸煞車信號 |  |
| 671 | M219_Servo.ON | 第三主軸伺服啟動 Servo on |  |
| 672 | M219Command | 第三主軸開始定位流程 Start flow |  |
| 718 | SP3_BRK_Sig | 第三主軸煞車信號 |  |
| 750 |  | 所有軸第一行程軟體行程極限切換(保留) | 立車 Ver.,Loader |
| 751 |  | X軸第一行程軟體行程極限切換(保留) | 立車 Ver.,Loader |
| 752 |  | Y軸第一行程軟體行程極限切換(保留) | 立車 Ver.,Loader |
| 753 |  | Z軸第一行程軟體行程極限切換(保留) | 立車 Ver.,Loader |
| 754 |  | CH4軸第一行程軟體行程極限切換(保留) | 立車 Ver.,Loader |
| 755 |  | CH5軸第一行程軟體行程極限切換(保留) | 立車 Ver.,Loader |
| 756 |  | CH6軸第一行程軟體行程極限切換(保留) | 立車 Ver.,Loader |
| 757 |  | 釋放第一軟體行程極限檢查(保留) | 立車 Ver.,Loader |
| 790 | En_LDR | 啟動Loader軸功能 |  |
| 791 | EM_LDR | (保留)Loader軸EM功能 |  |
| 792 | MANUAL_LDR | 啟用Loader 軸手動操作 |  |
| 800~899 |  | C800~C899保留給MLC AXIS CONTROL FUNCTION |  |

    - S BITS定義

S BIT 	 CNC MLC

| BIT # | SYMBOL | DESCRIPTION | NOTE |
| --- | --- | --- | --- |
| 000 | STL | CYCLE START LIGHT |  |
| 001 | SPL | FEED HOLD LIGHT |  |
| 002 | EDITL | EDIT MDOE LIGHT |  |
| 003 | MEML | MEM MODE LIGHT |  |
| 004 | MDIL | MDI MODE LIGHT |  |
| 005 | JOGL | JOG MODE LIGHT |  |
| 006 | INCJOGL | INC JOG MODE LIGHT |  |
| 007 | MPGL | MPG MODE LIGHT |  |
| 008 | HOMEL | HOME MODE LIGHT |  |
| 009 | OPRL | OPR MODE LIGHT |  |
| 010 | MREADY | CNC STATUS BLOCK STOP(M_READY) |  |
| 011 | BSTOP | CNC STATUS BLOCK STOP(B_STOP) | 2012.11.23 |
| 012 | MANABS2 | Inverse of Manual Absolute Signal |  |
| 013 | NA | RESERVED |  |
| 014 |  |  |  |
| 015 |  |  |  |
| 016 | ZP1X | X 1st POINT RETURN END |  |
| 017 | ZP1Y | Y 1st POINT RETURN END |  |
| 018 | ZP1Z | Z 1st POINT RETURN END |  |
| 019 | ZP14 | 4th 1st POINT RETURN END |  |
| 020 | ZP2X | X 2nd POINT RETURN END |  |
| 021 | ZP2Y | Y 2nd POINT RETURN END |  |
| 022 | ZP2Z | Z 2nd POINT RETURN END |  |
| 023 | ZP24 | 4th 2nd POINT RETURN END |  |
| 024 | ZP15 | 5th 1st POINT RETURN END |  |
| 025 | ZP25 | 5th 2nd POINT RETURN END |  |
| 026 | HOMEOK | CNC check X(Y)Z axis Home Ok status |  |
| 027 | NA | RESERVED |  |
| 028 | NA | RESERVED |  |
| 029 | MF | M CODE READ |  |
| 030 | DEN | DISTRIBUTION END |  |
| 031 | AL | ALARM |  |
| 032 | RST | RESET |  |
| 033 | MA | NC READY | 同於S533 |
| 034 | ALSOFF | ALARM FOR SERVO OFF |  |
| 035 | NA | RESERVED |  |
| 036 | INCFEED1 | INCJOG 1.*1, MPG 1.*1 |  |
| 037 | INCFEED2 | INCJOG 2.*10, MPG 2.*10 |  |
| 038 | INCFEED3 | INCJOG 3.*100, MPG 3.*100 |  |
| 039 | INCFEED4 | INCJOG 4.*1000, MPG 4.*1000 |  |
| 040 | SBKL | SINGLE BLOCK LIGHT |  |
| 041 | BDTL | OPTIONAL BLOCK SKIP LIGHT |  |
| 042 | DRNL | DRY RUN LIGHT |  |
| 043 | MLKL | MACHINE LOCK LIGHT |  |
| 044 | OPSL | OPTIONAL STOP LIGHT |  |
| 045 | RTL | RAPID TRAVEL LIGHT |  |
| 046 | ZNGL | Z AXIS CANCEL LIGHT |  |
| 047 | AFLL | AUXILIARY FUNCTION LOCK LIGHT |  |
| 048 | PCS | Parts counter sufficient |  |
| 049 |  |  |  |
| 050 |  |  |  |
| 051 |  |  |  |
| 052 |  |  |  |
| 053 |  |  |  |
| 054 | SF | S CODE READ |  |
| 055 |  |  |  |
| 056 |  |  |  |
| 057 |  |  |  |
| 058 |  |  |  |
| 059 | NA | RESERVED |  |
| 060 | SK1 | SOFT KEY 1 (Ex: Auto power off) |  |
| 061 | SK2 | SOFT KEY 2 (Ex: NA) |  |
| 062 | SK3 | SOFT KEY 3 (Ex: M18) |  |
| 063 | SK4 | SOFT KEY 4 (Ex: MST optional function) |  |
| 064 | SK5 | SOFT KEY 5 (Ex: Air blow) |  |
| 065 | SK6 | SOFT KEY 6 (Ex: M60) |  |
| 066 | SK7 | SOFT KEY 7 (Ex: M61) |  |
| 067 | SRVOFFL | SERVO OFF LIGHT(RESERVED) |  |
| 068 | MPGDRNL | MPG DRY RUN LIGHT |  |
| 069 | TF | T CODE READ |  |
| 070 | NA | RESERVED |  |
| 071 | NA | RESERVED |  |
| 072 | NA | RESERVED |  |
| 073 | NA | RESERVED |  |
| 074 | TSX | TORQUE SKIP X AXIS |  |
| 075 | TSY | TORQUE SKIP Y AXIS |  |
| 076 | TSZ | TORQUE SKIP Z AXIS |  |
| 077 | TS4 | TORQUE SKIP CH4 AXIS |  |
| 078 | TS5 | TORQUE SKIP CH5 AXIS |  |
| 079 | TS6 | TORQUE SKIP CH6 AXIS |  |
| 080 | M00 | PROGRAM STOP |  |
| 081 | M01 | OPTIONAL STOP |  |
| 082 | M02 | PROGRAM RESET |  |
| 083 | M30 | RESET & REWIND |  |
| 084 | MAIN_OPRCW | MAIN_OPR SPINDLE CW |  |
| 085 | MAIN_OPRCCW | MAIN_OPR SPINDLE CCW |  |
| 086 | ORTFIN | ORIENTATION FINISH(RESERVED) |  |
| 087 | ORTLSA | ORIENTATION LOW SPEED ARRIVAL(RESERVED) |  |
| 088 | MAIN_SPDARV | MAIN SPINDLE SPEED ARRIVAL |  |
| 089 | ORTM | ORIENTATION ONE MORE TURN(RESERVED) |  |
| 090 | MAIN_RTSR | RIGID TAPPING MAIN SPINDLE REVERSE FROM BOTTOM(RESERVED) |  |
| 091 | G80 | CANNED CYCLE CANCEL |  |
| 092 | MAIN_ZSA | MAIN SPINDLE ZERO SPEED ARRIVAL |  |
| 093 |  |  |  |
| 094 | MAIN_SVRONS2C | MAIN SERVO ON FOR SPINDLE CHANGE TO C AXIS |  |
| 095 | MAIN_SPDSAG | MAIN SPINDLE SPEED ARRIVAL FOR AUTO GEAR CHANGE FUNCTION |  |
| 096 | MAIN_LOWGEAR | MAIN SPINDLE LOW GEAR |  |
| 097 | MAIN_MIDGEAR | MAIN SPINDLE MIDDLEGEAR |  |
| 098 | MAIN_3RDGEAR | MAIN SPINDLE 3RD GEAR |  |
| 099 | MAIN_HIGHGEAR | MAIN SPINDLE HIGH GEAR |  |
| 100 | UO0 | MACRO OUTPUT 0 |  |
| 101 | UO1 | MACRO OUTPUT 1 |  |
| 102 | UO2 | MACRO OUTPUT 2 |  |
| 103 | UO3 | MACRO OUTPUT 3 |  |
| 104 | UO4 | MACRO OUTPUT 4 |  |
| 105 | UO5 | MACRO OUTPUT 5 |  |
| 106 | UO6 | MACRO OUTPUT 6 |  |
| 107 | UO7 | MACRO OUTPUT 7 |  |
| 108 | UO8 | MACRO OUTPUT 8 |  |
| 109 | UO9 | MACRO OUTPUT 9 |  |
| 110 | UO10 | MACRO OUTPUT 10 |  |
| 111 | UO11 | MACRO OUTPUT 11 |  |
| 112 | UO12 | MACRO OUTPUT 12 |  |
| 113 | UO13 | MACRO OUTPUT 13 |  |
| 114 | UO14 | MACRO OUTPUT 14 |  |
| 115 | UO15 | MACRO OUTPUT 15 |  |
| 116 |  |  |  |
| 117 |  |  |  |
| 118 |  |  |  |
| 119 |  |  |  |
| 120 | 保留 | PMC AXIS FINISH |  |
| 121 |  |  |  |
| 122 |  |  |  |
| 123 |  |  |  |
| 124 |  |  |  |
| 125 |  |  |  |
| 126 |  |  |  |
| 127 |  |  |  |
| 150 | CF2C1 | CS/CF AXIS CHANGE TO C AXIS COMMAND |  |
| 151 | 保留 | CF AXIS CHANGE TO SPINDLE COMMAND |  |
| 152 | ZP16 | 6th 1st POINT RETURN END |  |
| 153 | ZP26 | 6th 2nd POINT RETURN END |  |
| 162 | NCSD | NC to Shut Down Signal |  |
| 165 | MAIN_SRFTCA | Spindle Run for Tool Change Active |  |
| 166 | M19_Flow.FIN | 第一主軸定位動作結束(Flow Finish) |  |
| 167 | MAIN_InputVoltage | 0 to 10v s167=0 -10v to 10v s167=1 |  |
| 168 | M29_NC.FIN | 第二主軸定位動作結束(Flow Finish) |  |
| 169 | M29_HAND.FIN | 第二主軸定位結束溝通流程結束(Handshake Finish)。 |  |
| 170 | M19_HAND.FIN | 第一主軸定位結束溝通流程結束(Handshake Finish)。 |  |
| 180 | Mill_ Rev _Fin | 銑削軸反向Mcode 結束信號 |  |
| 181 | Mill _LOWGEAR | SUB SPINDLE LOW GEAR |  |
| 182 | Mill _MIDGEAR | SUB SPINDLE MIDDLEGEAR |  |
| 183 | Mill _3RDGEAR | SUB SPINDLE 3RD GEAR |  |
| 184 | Mill _HIGHGEAR | SUB SPINDLE HIGH GEAR |  |
| 200 | ST | Cycle Start | SoftPanel |
| 201 | HOLD | Feed Hold | SoftPanel |
| 202 | RESET | Reset | SoftPanel |
| 203 |  | (HMICNC) | SoftPanel |
| 204 |  | (HMICNC) | SoftPanel |
| 205 | POWER | Power | SoftPanel |
| 206 | LIGHT | Lighting | SoftPanel |
| 207 | AIR | Air Coolant | SoftPanel |
| 208 | SHOWER | Shower | SoftPanel |
| 209 | FLUSH | Flushing | SoftPanel |
| 210 | CTS | 刀具中心出水 | SoftPanel |
| 211 | DOOR | Door Open | SoftPanel |
| 212 | SBK | Single Block | SoftPanel |
| 213 | OSP | Option Stop | SoftPanel |
| 214 | BDT | Option Block Skip | SoftPanel |
| 215 | MLK | Machine Lock | SoftPanel |
| 216 | Z MLK | Z.Cancel | SoftPanel |
| 217 | MST // | M.S.T Lock | SoftPanel |
| 218 | DRN | Dry Run | SoftPanel |
| 219 | MPG DRN | MPG Dry Run | SoftPanel |
| 220 | ATC | Tool Change | SoftPanel |
| 221 | ORT | Spindle Orientation | SoftPanel |
| 222 |  | (HMICNC) | SoftPanel |
| 223 |  | (HMICNC) | SoftPanel |
| 224 |  | (HMICNC) | SoftPanel |
| 225 |  | (HMICNC) | SoftPanel |
| 226 |  | (HMICNC) | SoftPanel |
| 227 |  | (HMICNC) | SoftPanel |
| 228 |  | (HMICNC) | SoftPanel |
| 229 |  | (HMICNC) | SoftPanel |
| 230 | FEED - | Feed Overrate - | SoftPanel |
| 231 | FEED + | Feed Overrate + | SoftPanel |
| 232 | FEED 100% | Feed Overrate 100% | SoftPanel |
| 233 | RAPID - | Rapid Override - | SoftPanel |
| 234 | RAPID + | Rapid overrate + | SoftPanel |
| 235 | RAPID 100% | Rapid 100% | SoftPanel |
| 236 | SP - | Spindle overrate - | SoftPanel |
| 237 | SP + | Spindle overrate + | SoftPanel |
| 238 | SP 100% | Spindle overrate 100% | SoftPanel |
| 239 | MPG*1 | MPG Override X1 | SoftPanel |
| 240 | MPG*10 | MPG Override X10 | SoftPanel |
| 241 | MPG*100 | MPG Override X100 | SoftPanel |
| 242 | -X | -X Axis Direction | SoftPanel |
| 243 | +X | +X Axis Direction | SoftPanel |
| 244 | -Y | -Y Axis Direction | SoftPanel |
| 245 | +Y | +Y Axis Direction | SoftPanel |
| 246 | -Z | -Z Axis Direction | SoftPanel |
| 247 | +Z | +Z Axis Direction | SoftPanel |
| 248 | -4 | -4 Axis Direction | SoftPanel |
| 249 | +4 | +4 Axis Direction | SoftPanel |
| 250 | -5 | -5 Axis Direction | SoftPanel |
| 251 | +5 | +5 Axis Direction | SoftPanel |
| 280 | SP CW | Spindle Forward | SoftPanel |
| 281 | SP STP | Spindle Stop | SoftPanel |
| 282 | SP CCW | Spindle Reverse | SoftPanel |
| 283 | ATC CW | ATC motor cw | SoftPanel |
| 284 | ATC CCW | ATC motor ccw | SoftPanel |
| 285 | CHIP ON | 捲屑機正轉 | SoftPanel |
| 286 | CHIP REV | 捲屑機反轉 | SoftPanel |
| 287 | Water ON | Coolant Fluid On | SoftPanel |
| 288 | Water STP | Coolant Fluid Stop | SoftPanel |
| 289 | Water AUTO | Coolant Fluid Auto | SoftPanel |
| 290 | EDIT | EDIT Mode | SoftPanel |
| 291 | MEM | MEM Mode | SoftPanel |
| 292 | MDI | MDI Mode | SoftPanel |
| 293 | JOG | Jog Mode | SoftPanel |
| 294 | RAPID | Rapid Mode(inc jog) | SoftPanel |
| 295 | MPG | MPG Mode | SoftPanel |
| 296 | HOME | Home Mode | SoftPanel |
| 297 | OPR | OPER Mode | SoftPanel |
| 298 |  | (HMICNC) | SoftPanel |
| 299 |  | (HMICNC) | SoftPanel |
| 300 |  |  |  |
| 400 | DG_SVO_RDY | Digital Driver Servo Ready (Break Signal) |  |
| 401 |  | 第1軸向移動中 |  |
| 402 |  | 第2軸向移動中 |  |
| 403 |  | 第3軸向移動中 |  |
| 404 |  | 第4軸向移動中 |  |
| 405 |  | 第5軸向移動中 |  |
| 406 |  | 第6軸向移動中 |  |
| 407 |  | 第7軸向移動中 |  |
| 408 |  | 第8軸向移動中 |  |
| 409 |  | 第9軸向移動中 |  |
| 410 |  | 第10軸向移動中 |  |
| 411 |  | 第11軸向移動中 |  |
| 412 |  | 第12軸向移動中 |  |
| 413 | CNC_MOT_RDY | MOT位置維護備妥信號 |  |
| 414 | 保留 |  |  |
| 500 | MAIN_HOME_COMP | MAIN Spindle return home completion |  |
| 501 | SUB_HOME_COMP | SUB Spindle return home completion |  |
| 502 | MAIN_CHUK | MAIN Spindle chuck ON/OFF |  |
| 503 | SUB_CHUK | SUB Spindle chuck ON/OFF |  |
| 504 | SPSYC | Spindle synchronous command |  |
| 505 | SPPHS | Spindle phase synchronous command |  |
| 506 | FSPSY | Spindle synchronous completion signal |  |
| 507 | FSPPH | Spindle phase synchronous completion signal |  |
| 508 | RLS_ SPSYC | Release Spindle speed synchronization |  |
| 509 | RLS_ SPPHS | Release Spindle phase synchronization |  |
| 510 | SUB_OPRCW | SUB Spindle OPR SPINDLE CW |  |
| 511 | SUB_OPRCCW | SUB Spindle OPR SPINDLE CCW |  |
| 512 | SUB_SPDARV | SUB SPINDLE SPEED ARRIVAL |  |
| 513 | SUB_RTSR | RIGID TAPPING SUB SPINDLE REVERSE FROM BOTTOM |  |
| 514 | SUB_ZSA | SUB SPINDLE ZERO SPEED ARRIVAL |  |
| 515 | SUB_SVRONS2C | SERVO ON FOR SUB SPINDLE CHANGE TO C2 AXIS |  |
| 516 | SUB_SPDSAG | SUB SPINDLE SPEED ARRIVAL FOR AUTO GEAR CHANGE FUNCTION |  |
| 517 | SUB_LOWGEAR | SUB SPINDLE LOW GEAR |  |
| 518 | SUB_MIDGEAR | SUB SPINDLE MIDDLEGEAR |  |
| 519 | SUB_3RDGEAR | SUB SPINDLE 3RD GEAR |  |
| 520 | SUB_HIGHGEAR | SUB SPINDLE HIGH GEAR |  |
| 521 | CF_2C2 | CF AXIS CHANGE TO C2 AXIS COMMAND |  |
| 522 | CF_2SUB | CF AXIS CHANGE TO SUB SPINDLE COMMAND |  |
| 523 | SUB_SRFTCA | SUB Spindle Run for Tool Change Active |  |
| 524 | SUB_M119_NC.FIN | SUB Spindle M119_NC.FINISH(主軸定位完成) |  |
| 525 | SUB_InputVoltage | 0 to 10v s525=0 -10v to 10v s525=1 |  |
| 527 | MAIN_SUB_CTRL_SEL | Main or Sub Spindle which is controlled by system(OFF:Main,ON:Sub) |  |
| 528 | MAIN_SP_DR_ALM | Main spindle driver alarm |  |
| 529 | MILL_SP_DR_ALM | Mill spindle driver alarm |  |
| 530 | SUB_SP_DR_ALM | Sub spindle driver alarm |  |
| 531 | MAIN_SP_ST_FIN | Main spindle state change finish | 2014.8.1 Add |
| 532 | SUB_SP_ST_FIN | Sub spindle state change finish | 2014.8.1 Add |
| 533 | CNC_SYS_RDY | CNC system ready signal | 2014.9.15 Add |
| 534 | CNC_SYN_SEIZE_FAIL | Synchronization fail when two chucks seize same workpiece | 2016.10.14 |
| 535 | CNC_SP_PARA | SP Parameters (OFF:SYN parameters ON:SP parameters) | 2016.12.15 |
| 540 | SMP_SP_SYNC_FIN | 簡易主軸同期完成 |  |
| 541 | SMP_SP_SYNC_LERR | 簡易主軸同期誤差過大 |  |
| 550 | CNC_SV_SYNMT1 | Machine Coordinate match state |  |
| 551 | CNC_SV_SYNMT2 | Machine Coordinate match state |  |
| 552 | CNC_SV_SYNMT3 | Machine Coordinate match state |  |
| 553 | CNC_SV_SYNMT4 | Machine Coordinate match state |  |
| 554 | CNC_SV_SYNMT5 | Machine Coordinate match state |  |
| 555 | CNC_SV_SYNMT6 | Machine Coordinate match state |  |
| 556 | CNC_SV_SYNOF1 | Synchronization compensation enable state |  |
| 557 | CNC_SV_SYNOF2 | Synchronization compensation enable state |  |
| 558 | CNC_SV_SYNOF3 | Synchronization compensation enable state |  |
| 559 | CNC_SV_SYNOF4 | Synchronization compensation enable state |  |
| 560 | CNC_SV_SYNOF5 | Synchronization compensation enable state |  |
| 561 | CNC_SV_SYNOF6 | Synchronization compensation enable state |  |
| 562 | CNC_SV_SYNER | Positional deviation error alarm |  |
| 563 | CNC_SV_SYNO1 | Axis Synchronous Control Signal |  |
| 564 | CNC_SV_SYNO2 | Axis Synchronous Control Signal |  |
| 565 | CNC_SV_SYNO3 | Axis Synchronous Control Signal |  |
| 566 | CNC_SV_SYNO4 | Axis Synchronous Control Signal |  |
| 567 | CNC_SV_SYNO5 | Axis Synchronous Control Signal |  |
| 568 | CNC_SV_SYNO6 | Axis Synchronous Control Signal |  |
| 570 | LNCMOD | 學習模式確認 |  |
| 571 | LNCING | 學習忽略確認 |  |
| 572 | LNACT | 學習控制執行中 |  |
| 573 | LNERR | 學習誤差過大 |  |
| 574 | LNFIN | 學習結束 |  |
| 575 | LNSTP | 學習停止 |  |
| 666 | M219_NC.FIN | 第三主軸定位動作結束(Flow Finish) |  |
| 670 | M219_HAND.FIN | 第三主軸定位結束溝通流程結束(Handshake Finish)。 |  |
| 701 |  | 保留給外部計時器1使用開 |  |
| 702 |  | 保留給外部計時器1使用關 |  |
| 703 |  | 保留給外部計時器2使用開 |  |
| 704 |  | 保留給外部計時器2使用關 |  |
| 705 |  | 保留給外部計時器3使用開 |  |
| 706 |  | 保留給外部計時器3使用關 |  |
| 707 |  | 保留給外部計時器4使用開 |  |
| 708 |  | 保留給外部計時器4使用關 |  |
| 709 |  | 保留給外部計時器5使用開 |  |
| 710 |  | 保留給外部計時器5使用關 |  |
| 711 |  | 保留給外部計時器6使用開 |  |
| 712 |  | 保留給外部計時器6使用關 |  |
| 713 |  | 保留給外部計時器7使用開 |  |
| 714 |  | 保留給外部計時器7使用關 |  |
| 715 |  | 保留給外部計時器8使用開 |  |
| 716 |  | 保留給外部計時器8使用關 |  |
| 717 |  | 保留給外部計時器9使用開 |  |
| 718 |  | 保留給外部計時器9使用關 |  |
| 719 |  | 保留給外部計時器10使用開 |  |
| 720 |  | 保留給外部計時器10使用關 |  |
| 731 |  | X軸移動中 等速移動信號 |  |
| 732 |  | Y軸移動中 等速移動信號 |  |
| 733 |  | Z軸移動中 等速移動信號 |  |
| 734 |  | CH4軸移動中 等速移動信號 |  |
| 735 |  | CH5軸移動中 等速移動信號 |  |
| 736 |  | CH6軸移動中 等速移動信號 |  |
| 737 |  | X軸移動中 加減速移動信號 |  |
| 738 |  | Y軸移動中 加減速移動信號 |  |
| 739 |  | Z軸移動中 加減速移動信號 |  |
| 740 |  | CH4軸移動中 加減速移動信號 |  |
| 741 |  | CH5軸移動中 加減速移動信號 |  |
| 742 |  | CH6軸移動中 加減速移動信號 |  |
| 751 |  | X軸到達第一軟體行程極限(保留) | 立車 Ver.,Loader |
| 752 |  | Y軸到達第一軟體行程極限(保留) | 立車 Ver.,Loader |
| 753 |  | Z軸到達第一軟體行程極限(保留) | 立車 Ver.,Loader |
| 754 |  | CH4軸到達第一軟體行程極限(保留) | 立車 Ver.,Loader |
| 755 |  | CH5軸到達第一軟體行程極限(保留) | 立車 Ver.,Loader |
| 756 |  | CH6軸到達第一軟體行程極限(保留) | 立車 Ver.,Loader |
| 761 | CNC_1_SW_IN_RANGE | 到達第1個位置開關視窗範圍(保留) | 立車 Ver.,Loader |
| 762 | CNC_2_SW_IN_RANGE | 到達第2個位置開關視窗範圍(保留) | 立車 Ver.,Loader |
| 763 | CNC_3_SW_IN_RANGE | 到達第3個位置開關視窗範圍(保留) | 立車 Ver.,Loader |
| 764 | CNC_4_SW_IN_RANGE | 到達第4個位置開關視窗範圍(保留) | 立車 Ver.,Loader |
| 765 | CNC_5_SW_IN_RANGE | 到達第5個位置開關視窗範圍(保留) | 立車 Ver.,Loader |
| 766 | CNC_6_SW_IN_RANGE | 到達第6個位置開關視窗範圍(保留) | 立車 Ver.,Loader |
| 767 | CNC_7_SW_IN_RANGE | 到達第7個位置開關視窗範圍(保留) | 立車 Ver.,Loader |
| 768 | CNC_8_SW_IN_RANGE | 到達第8個位置開關視窗範圍(保留) | 立車 Ver.,Loader |
| 769 | CNC_9_SW_IN_RANGE | 到達第9個位置開關視窗範圍(保留) | 立車 Ver.,Loader |
| 770 | CNC_10_SW_IN_RANGE | 到達第10個位置開關視窗範圍(保留) | 立車 Ver.,Loader |
| 771 | CNC_11_SW_IN_RANGE | 到達第11個位置開關視窗範圍(保留) | 立車 Ver.,Loader |
| 772 | CNC_12_SW_IN_RANGE | 到達第12個位置開關視窗範圍(保留) | 立車 Ver.,Loader |
| 773 | CNC_13_SW_IN_RANGE | 到達第13個位置開關視窗範圍(保留) | 立車 Ver.,Loader |
| 774 | CNC_14_SW_IN_RANGE | 到達第14個位置開關視窗範圍(保留) | 立車 Ver.,Loader |
| 775 | CNC_15_SW_IN_RANGE | 到達第15個位置開關視窗範圍(保留) | 立車 Ver.,Loader |
| 776 | CNC_16_SW_IN_RANGE | 到達第16個位置開關視窗範圍(保留) | 立車 Ver.,Loader |
| 777 | CNC_17_SW_IN_RANGE | 到達第17個位置開關視窗範圍(保留) | 立車 Ver.,Loader |
| 778 | CNC_18_SW_IN_RANGE | 到達第18個位置開關視窗範圍(保留) | 立車 Ver.,Loader |
| 779 | CNC_19_SW_IN_RANGE | 到達第19個位置開關視窗範圍(保留) | 立車 Ver.,Loader |
| 780 | CNC_20_SW_IN_RANGE | 到達第20個位置開關視窗範圍(保留) | 立車 Ver.,Loader |
| 781 | CNC_21_SW_IN_RANGE | 到達第21個位置開關視窗範圍(保留) | 立車 Ver.,Loader |
| 782 | CNC_22_SW_IN_RANGE | 到達第22個位置開關視窗範圍(保留) | 立車 Ver.,Loader |
| 783 | CNC_23_SW_IN_RANGE | 到達第23個位置開關視窗範圍(保留) | 立車 Ver.,Loader |
| 784 | CNC_24_SW_IN_RANGE | 到達第24個位置開關視窗範圍(保留) | 立車 Ver.,Loader |
| 790 | Loader功能啟用 | 成功啟用Loader軸功能 |  |
| 800~999 |  | S800~S999保留給MLC AXIS CONTROL FUNCTION |  |

    - R暫存器

REGISTER  (R000~R036)

| REG # | SYMBOL | DESCRIPTION | NOTE |
| --- | --- | --- | --- |
| 000 |  |  |  |
| 001 | M_CODE | M00~M99 |  |
| 002 | S_CODE | S000~S9999 | $109 |
| 003 | T_CODE | T000~T9999 | $112 |
| 004 | Reserved | Reserved, CNC system used. |  |
| 005 | Reserved | Reserved, CNC system used. |  |
| 006 | PCS | Parts counter |  |
| 007 |  |  |  |
| 008 |  |  |  |
| 009 |  |  |  |
| 010 |  |  |  |
| 011 |  |  |  |
| 012 |  |  |  |
| 013 | OPMODE | 1.EDIT 2.AUTO 3.MDI 4. JOG 5.INCJOG 6.MPG 7.HOME 8.OPR |  |
| 014 | INCFED | INCJOG 1.*1 2.*10 3.*100 4.*1000 MPG 1.*1 2.*10 3.*100 4.*100 |  |
| 015 | SPDOV | 1.50% 2.60% 3.70% 4.80% 5.90% 6.100% 7.110% 8.120% |  |
| 016 | FEEDOV | 0.0% 1.10% 2.20% 3.30% 4.40% 5.50% ~ 15.150% |  |
| 017 | JOGOV | 0.0% 1.10% 2.20% 3.30% 4.40% 5.50% ~ 15.150% |  |
| 018 | RTOV | 1. F0 2.25% 3.50% 4.100% |  |
| 019 |  |  |  |
| 020 | PMCD | PMC DISTANCE |  |
| 021 | PMCF | PMC FEEDRATE |  |
| 022 | PMCC | PMC CONTROL |  |
| 023 | PMCS | PMC SPINDLE |  |
| 024 |  |  |  |
| 039 | ATCSTEP | ATC STEP NUMBER |  |
| 040 | OPMES1 | MESSAGE #01~#16 |  |
| 041 | OPMES2 | MESSAGE #17~#32 |  |
| 042 | OPMES3 | MESSAGE #33~#48 |  |
| 043 | OPMES4 | MESSAGE #49~#64 |  |
| 044 | OPMES5 | MESSAGE #65~#80 |  |
| 045 | OPMES6 | MESSAGE #81~#96 |  |
| 046 | OPMES7 | MESSAGE #97~#112 |  |
| 047 | OPMES8 | MESSAGE #113~#128 |  |
| 048 | OPMES9 | MESSAGE #129~#144 |  |
| 049 | OPMES10 | MESSAGE #145~#160 |  |
| 050 | OPMES11 | MESSAGE #161~#176 |  |
| 124 |  |  | $113 |
| 125 |  |  | $114 |
| 301 | X-machine | X軸機械座標值(最小單位) |  |
| 302 | Y- machine | Y軸機械座標值(最小單位) |  |
| 303 | Z- machine | Z軸機械座標值(最小單位) |  |
| 304 | CH4- machine | CH4軸機械座標值(最小單位) |  |
| 305 | CH5- machine | CH5軸機械座標值(最小單位) |  |
| 306 | CH6- machine | CH6軸機械座標值(最小單位) |  |
| 400~499 |  | R400~R499保留給MLC AXIS CONTROL FUNCTION |  |

- 銑床與手臂IO表